Phils Design Tools - User Guide

Purpose

This guide explains how to use the Phils Design Tools add-in in Fusion 360 to go from wireframe to fabrication-ready steel members.

Prerequisites

- Fusion 360 installed.

- Add-in installed from Addin\PhilsDesignTools and enabled.

- Wireframe DXF exported from Rhino, with sectors and member types split by layer names.

Workflow Overview

1. Locate steel wireframe model (DXF).

2. Import wireframe into Fusion 360, organized by sector components.

3. Generate RHS/SHS columns and arms sector by sector.

4. Trim member ends where brackets will sit (thickness + 3 mm clearance).

5. Place brackets from the Fusion library.

6. Project bracket outlines to create EA sketch lines.

7. Generate EA members for bracket connections and rotate as needed.

8. Create cross-bracing sketches (flat to faces).

9. Generate EA cross-bracing and orient to face.

10. Cut bolt holes.

11. Rename all components to the naming convention.

12. Repeat for remaining sectors.

13. Full design check (alignment, holes, naming).

14. Export IGES for RHS/SHS components.

15. Export EA hole list for manufacturing.

Using the Tools

EA From Lines

- Select sketch lines for EA members.

- Set flange length, thickness, hole size, hole gauge, and extra end.

- Set orientation to 0, 90, 180, or 270 degrees.

SHS From Lines

- Select sketch lines for SHS members.

- Set size, wall thickness, and extra end.

- Set orientation to 0, 90, 180, or 270 degrees.

RHS From Lines

- Select sketch lines for RHS members.

- Set width, depth, wall thickness, and extra end.

- Set orientation to 0, 90, 180, or 270 degrees.

Stub Arms To Wall

- Select a column face for manual control, or select a column body/occurrence for auto face selection.

- Select wall faces, bodies, or occurrences.

- Set connection points, bottom/top offsets, and wall clearance (default 200 mm).

- Lines are created only when the hit lands on a selected wall face boundary.

- Stub arm sketches are stored under the "stub arm lines" component; guide lines and wall sketches are not kept.

Rotate Steel Member

- Select one or more member occurrences.

- Choose +90 or -90 rotation.

- EA rotates about the hole axis; SHS/RHS rotates about its length axis.

EA Batch Rename

- Select member occurrences to rename.

- Enter prefix (type + index), size suffix, and starting index.

- The tool finds the longest edge as the length.

Naming Convention

Format: SectorLetter-TypeWithIndex-Size-Lengthmm

Examples:

- P-C1-100x50x3-2400mm

- I-CA2-100x50x3-1800mm

- S-EAH5-50x50x3-600mm

Notes and Tips

- Work sector by sector to keep naming consistent.

- Use rotation presets to align members quickly.

- For trim and bracket placement, confirm the correct end before cutting.

Troubleshooting and Logs

- Logs are written to Addin\PhilsDesignTools\PhilsDesignTools.log.

- If a command fails, check the log for a stack trace.